Program to transfer first file (unlynx), save this program to disk with :-

'SAVE "TRANSFER1",8 (OR 1)

then type "RUN" on c64

make sure paralell lead is made and connected (see below)

make sure you are in cli shell and in the right directory with the un-lha'ed

files from the main archive

then type '64 aunlynx.c64' on amiga,wait for amiga to finish (back 2

amigados prompt)

press "C=" key on C64 (bottom left) (don't press before amiga has finished or

bytes will be missed out!!)

press disk drive device number on C64 then type

'64 allfiles.lnx' to save them to c64 disk.

10 FOR A=820 TO 900:READB:POKEA,B:NEXTA:SYS820

20 DATA 160,0,140,3,221,169,63,141

30 DATA 2,221,173,0,221,41,251,141

40 DATA 0,221,120,173,1,221,230,1

50 DATA 153,252,7,198,1,200,208,3

60 DATA 238,78,3,173,0,221,41,251

70 DATA 141,0,221,173,1,220,201

80 DATA 223,208,21,173,77,3,133

90 DATA 248,173,78,3,133,249

100 DATA 169,0,141,0,8,32,89,166

110 DATA 76,174,167,169,16,44,13

120 DATA 221,240,221,208,195,189

Alternative entry

Just download the file 'aunlynx.c64' to the c64, run it from disk or

wotever. Then enter device number, type '64 allfiles.lnx' on Amiga.

Away ya go.

Make sure paralell lead is connected etc.

Schematics for Paralell port lead:

(Read the section on making a lead in ALZ64V10.DOC, this tells you of

any warnings about blowing up your Amiga... don't worry about blowing

up the C64, you can't... they're invincible...)

You need dis:

- One card edge connector (C64 user port reset switch cheapest)

- One male D-25 connector (Amiga parallel port)

- Suitable lenght of flat cable (Width 12 or more)

- A soldering iron and some solder

Additionally, you'll need the following files for your Amiga...

(I guess this shouldn't be too big problem as these are system files!)

- Devs/Parallel.device

- L/Port-handler

4. PData cable schematics

C64 user port              Amiga parallel port

Pin      Description       Description      Pin

B        FLAG*             STROBE           1

C        PB0               DATA 0           2

D        PB1               DATA 1           3

E        PB2               DATA 2           4

F        PB3               DATA 3           5

H        PB4               DATA 4           6

J        PB5               DATA 5           7

K        PB6               DATA 6           8

L        PB7               DATA 7           9

M        PA2               BUSY             11

8        PC*               ACK*             10

N        GND               GND              17

These paralell port lead instructions from Xdata/Pdata docs. Thanks.